Appendix

- Stability-Aware Metrics – SNM and Write Margin

Objective

- To evaluate how reliably the SRAM cell retains and flips stored values under noise, voltage fluctuations, and low-VDD operation.

- To integrate stability-aware metrics, which are Static Noise Margin (SNM), and Write Margin (WM)

- To ensure DSE results are also functionally safe, especially critical when operating at low VDD, where SNM collapses

Importance

SRAM designs can still fail silently if:

- SNM is too low during a read, leading to read disturb

- Write Margin is too narrow, causing write failure

Methodology

- Static Noise Margin (SNM)

- Simulate butterfly curves from cross-coupled inverters

- Measure maximum square that fits between them

- Step:

- Sweep input of inverter A

- Feed output to inverter B

- Plot VTC of A and mirrored B: measure maximum square

- Write Margin (WM)

Integration into DSE